Practical 04 Digital Presentation

Representing colour in RGB

- Go to web site https://www.w3schools.com/colors/colors_picker.asp

| Colour | RGB in decimal | RGB In Hex |
| --- | --- | --- |
| Black | 0, 0, 0 | #000000 |
| White | 255, 255, 255 | #ffffff |
| Maroon | 128, 0, 0 | #800000 |

Access the File Header

- Using notepad, create a text file named “file1.txt”

- Check the file properties

What is the size of the file in bytes?

18 bytes

- Visit HexEd at https://hexed.it/

or download HxD Hex Editor

https://download.cnet.com/HxD-Hex-Editor/3000-2352-10891068.html?part=dl-HxDHexEdi&subj=uo&tag=button

- Open file1.txt using HxD

- Observe the Hex code

| Character | Hex | Binary(8 bit) |
| --- | --- | --- |
| I | 49 | 01001001 |

- Using HxD to observe how character “o” and space character are represented

| Character | Hex | Binary(8 bit) |
| --- | --- | --- |
| o | 6F | 01101111 |
| space |  | 01001001 |

- Download the following image and save it as zebra.jpg, zebra.gif and zebra.bmp

- Open zebra.gif in HxD

Observe the Hex code

What are the first 6 bytes decoded?

| offset | 00 | 01 | 02 | 03 | 04 | 05 |
| --- | --- | --- | --- | --- | --- | --- |
| Byte | 01000111 | 01001001 | 01000110 | 00111000 | 00111001 | 01100001 |
| Character | 47 | 49 | 46 | 38 | 39 | 61 |

- Visit web site https://en.wikipedia.org/wiki/List_of_file_signatures to find out the meaning of above hex code

Image file encoded in the Graphics Interchange Format (GIF)

- Open zebra.jpg in HxD

Observe the Hex code

What are the first 12 bytes?

| offset | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 0A | 0B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 11111111 | 11011000 | 11111111 | 11100000 | 00000000 | 00010000 | 01100000 | 00000000 | 01100000 | 00000000 | 00000000 | 11011011 |
| Character | FF | D8 | FF | E0 | 00 | 10 | 4A | 00 | 60 | 00 | 00 | DB |

- Visit web site https://en.wikipedia.org/wiki/List_of_file_signatures to find out the meaning of above hex code

JPEG raw or in the JFIF or Exif file format

- Open zebra.bmp in hex editor

Observe the Hex code

What are the first 2 bytes?

| offset | 00 | 01 |
| --- | --- | --- |
| Byte | 01000010 | 01001101 |
| Character | 42 | 4D |

- Visit web site https://en.wikipedia.org/wiki/List_of_file_signatures to find out the meaning of above hex code

BMP file, a bitmap format used mostly in the Windows world

- (optional)
Download the file at https://drive.google.com/open?id=1vf0SBkI1LdWQvYmK9yM5AyJ0fHKE6DHR

How can you view it as an image?

Rename it as a jpg and save it as all files

Hint:

- Open the file using HxD

- Visit web site https://en.wikipedia.org/wiki/List_of_file_signatures to find out the meaning of above hex code

- Rename the extension of the file

End of Practical